*****************************************************************************

*           ffp68343 -- Fast Floating-Point Support for Fargo II            *

*                 Code Adapted to Fargo II by Aaron Hill                    *

*****************************************************************************

*                   68343 Fast Floating-Point Firmware                      *

*                        Release 1.2 July 29, 1983                          *

*               Copyright (C) 1980,1981,1982 Motorola, Inc.                 *

*****************************************************************************

TABLE OF CONTENTS:

I. Description

II. Contact Information

III. Bugs

IV. Formats

A. ASCII string

B. DB (Dual-Binary)

C. BCD (Binary-Coded Decimal)

V. Function Explaination

A. FFPABS

B. FFPNEG

C. FFPADD/FFPSUB

D. FFPAFP

E. FFPARND

F. FFPATAN

G. FFPCMP

H. FFPTST

I. FFPDBF

J. FFPDIV

K. FFPEXP

L. FFPFPA

M. FFPFPBCD

N. FFPFPI

O. FFPIFP

P. FFPLOG

Q. FFPMUL/FFPMUL2

R. FFPPWR

S. FFPSIN/FFPCOS/FFPTAN/FFPSINCS

T. FFPCOSH/FFPSINH/FFPTANH

U. FFPSQRT

I. DESCRIPTION:

ffp68343 is a library contianing the 68343 Fast Floating-Point Firmware

created by Motorola for the MC68000 and MC68010.  I have taken the original

source code, which is available on Motorola's FTP site, and converted it to

compile for Fargo II.  The actual algorithms have not been changed, and so

I must give full credit to Motorola.  The TI-ROM must have floating-point

routines implimented, but this library gives a nice alternative while David

searches the ROM.

The original software distribution contains floating-point support for

Motorola's FFP format and the IEEE standard (Draft 8.0).  Upon inspection

of the algorithms, I found that the IEEE functions merely converted to FFP

and used the FFP functions.  Although some may wish to use the IEEE format,

these functions are slower.  I have not included any IEEE functions in this

library.  Motorola also included two sample programs in the distribution.

There was a demo which tested various functions; a calculator was also

included.  I have converted the demo to test the functionality on several

ROMs.  If anyone is interested, I can give them the original source to the

calculator to convert.

II. CONTACT INFORMATION:

NAME: Aaron Hill

EMAIL: mozart@inlink.com

NICKNAME: SeracOhw (EF-NET)

III. BUGS:

As for bugs in the code, Motorola does not show any.  I found one bug

when compiling.  There was an alignment error in the FFPARND (Fast Floating-

Point ASCII Round) routine.  Here is the original code:

CMP.W   #'01',-1(a0)

This is a problem as you cannot reference a word on an odd address, but if

a0 pointed to an odd address then the -1 would make it even.  The compiler

would not let me get away with this.  So, I changed it to the following:

CMP.B   #'01',-1(a0)

If anyone has problems with the FFPARND routine, please notify me.  The

problem would most likely be this command.

IV. FORMATS:

Motorola does not include the FFP format in documentation.  I have tried

to analyze the source but cannot determine the format.  The firmware does

give you the ability to convert ASCII strings, DB (Dual-Binary), and integers

to FFP floats.  The documentation does explain the formats for the ASCII and

DB.

A.  ASCII String Format:

{SIGN}{DIGITS}{'.'}{DIGITS}{'E'}{SIGN}{DIGITS}

<*********MANTISSA********><*****EXPONENT****>

Syntax Rules:

* Both signs are optional and are '+' or '-'.

* The mantissa must be present

* The exponent need not be present

* The mantissa may lead with a decimal point

* The mantissa need not have a decimal point

Examples:  (All equal one-hundred-twenty)

120            .120E3

120.          +.120E+03

+120.          0.000120E6

0000120.00  1200000E-4

1200000.00E-0004

B.  DB (Dual-Binary) Format:

The FFPDBF routines uses D6 and D7 for input.

D6 Bit #16 - Represents Sign (0-Pos, 1-Neg)

D6.W       - Represents Base Ten Exponent

Considering D7 A Binary Integer

D7         - Binary Integer Mantissa

The firmware can convert FFP floats to ASCII strings, BCD and integers.

C.  BCD (Binary-Coded Decimal) Format:

MMMM S E S B

Where  M - Four bytes of BCD, each with two digits of the mantissa

S - Sign of mantissa ($00 +, $FF -)

E - BCD byte for two digit exponent

S - Sign of exponent ($00 +, $FF -)

B - One byte binary 2's complement representation of exponent

Examples   M M M M S E S B

$1200000000030003        +120

$3145926700010001        3.14159

$12000000FF030003        -120

$10000000FF10000A        -1,000,000,000

$100000000002FFFE        .001 (.1E-2)

M M M M S E S B

V. FUNCTION EXPLAINATION:

The include file, ffp68343.h, contains a quick reference for all of the

functions.  Included in the reference are the input and output arguments,

condition codes, and register status.  This explaination following includes

any information not in the header file.

A. FFPABS -- Fast Floating-Point Absolute Value

No extra information.

B. FFPNEG -- Fast Floating-Point Negate

No extra information.

C. FFPADD/FFPSUB -- Fast Floating-Point Add/Subtract

notes:

1) Addend/Subtrahend unaltered (d6).

2) Underflow returns zero and is unflagged.

3) Overflow returns the highest value with the

correct sign and the 'v' bit set in the ccr.

time: (8 mhz no wait states assumed)

composite average  20.625 microseconds

add:

arg1=0           7.75  microseconds

arg2=0           5.25  microseconds

like signs  14.50 - 26.00  microseconds

average          18.00  microseconds

unlike signs  20.13 - 54.38  microseconds

average          22.00  microseconds

subtract:

arg1=0           4.25  microseconds

arg2=0           9.88  microseconds

like signs  15.75 - 27.25  microseconds

average          19.25  microseconds

unlike signs  21.38 - 55.63  microseconds

average          23.25  microseconds

D. FFPAFP -- ASCII to Floating-Point

See FORMATS section for format.

floating point range:

fast floating point supports the value zero and

non-zero value within the following bounds -

18                             -20

9.22337177 x 10   > +number >  5.42101070 x 10

18                             -20

-9.22337177 x 10   > -number > -2.71050535 x 10

precision:

this conversion results in a 24 bit precision with

guaranteed error less than or equal to one-half

least significant bit.

notes:

1) this routine calls the dual-binary to float routine and

can be used as an illustration of how to 'front-end' that

routine with a customized scanner

2) underflows return a zero without any indicators set

3) overflows will return the maximum value possible with

proper sign and the 'b' bit set in the ccr

4) if the 'c' bit in the ccr indicates an invalid pattern

detected, then a0 will point to the invalid character

logic summary:

a) process leading sign

b) process pre-decimalpoint digits and increment 10 power

bias for each digit bypassed due to 32 bit overflow

c) process post-decimalpoint digits decrementing the 10 power

bias for each

d) process the exponent

e) add the 10 power bias to the exponent

f) call 'ffpdbf' routine to finish conversion

time: (8 mhz no wait states)

374 microseconds converting the string

E. FFPARND -- ASCII Round

This routine is normally called after the FFPFPA float to

ascii routine and acts upon its results.

The rounding precision represents the power of ten to which

the rounding will occur.  (i.e. a -2 means round the digit

in the hundredth position for resultant rounding to tenths.)

A positive value indicates rounding to the left of the decimal

point (0 is units, 1 is tens e.t.c.)

The base ten exponent in binary is d7 from the FFPFPA routine

or computed by the caller.

The stack contains the return address followed by the ascii

number as from the 'ffpfpa' routine.  see the description of

that routine for the required format.                        *

example: (input pattern '+.98765432+01' = 9.8765432)

round +1 is +.00000000+00 =  0.

round  0 is +.10000000+02 = 10.

round -1 is +.10000000+02 = 10.

round -2 is +.99000000+01 =  9.9

round -3 is +.98800000+01 =  9.88

round -6 is +.98765400+01 =  9.87654

notes:

1) if the rounding digit is to the left of the most significant

digit, a zero results.  if the rounding digit is to the right

of the least significant digit, then no rounding occurs

2) rounding is handy for eliminating the dangling '999...' problem

common with float to decimal conversions.

3) positions from the rounded digit and to the right are set to

zeroes.

4) the exponent may be affected.

5) rounding is forced by adding five.

6) the binary exponent in d7 may be pre-biased by the caller to

provide enhanced editing control.

7) the return address is removed from the stack upon exit.

F. FFPATAN -- Fast Floating-Point Arctangent

notes:

1) spot checks show at least six digit precision on all

sampled cases.

time: (8mhz no wait states assumed)

the time is very data sensitive with sample values ranging

from 238 to 465 microseconds

G. FFPCMP -- Fast Floating-Point Compare

No extra information.

H. FFPTST -- Fast Floating-Point Test

No extra information.

I. FFPDBF -- Dual-Binary to Float

floating point range:

fast floating point supports the value zero and non-zero

values within the following bounds -

base 10

18                             -20

9.22337177 x 10   > +number >  5.42101070 x 10

18                             -20

-9.22337177 x 10   > -number > -2.71050535 x 10

base 2

63                           -63

.ffffff  x  2   > +number >  .ffffff  x  2

63                           -64

-.ffffff  x  2   > -number > -.ffffff  x  2

precision:

this conversion results in a 24 bit precision with

guaranteed error less than or equal to one-half least

significant bit.

notes:

1) the input formats have been designed for ease of parsing

text for conversion to floating point.  see ffpasf for

comments describing the method for setup to this routine.

2) underflows return a zero without any indicators set.

3) overflows will return the maximum value possible with proper

sign and the 'v' bit set in the ccr register.

J. FFPDIV -- Fast Floating-Point Divide

notes:

1) divisor is unaltered (d6).

2) underflows return zero without any indicators set.

3) overflows return the highest value with the proper sign and

'v' bit set in the ccr.

4) if a divide by zero is attempted the divide by zero exception

trap is forced by this code with the original arguments intact.

if the exception returns with the denominator altered the

divide operation continues, otherwise an overflow is forced

with the proper sign. the floating divide by zero can be

distinguished from true zero divide by the fact that it is an

immediate zero dividing into register d7.

time: (8 mhz no wait states assumed)

dividend zero         5.250 microseconds

minimum time others  72.750 microseconds

maximum time others  85.000 microseconds

average others       76.687 microseconds

K. FFPEXP -- Fast Floating-Point Exponent

notes:

1) an overflow returns the largest magnitude number.

2) spot checks show at least 6.8 digit accuracy for

all abs(arg) < 30.

time: (8mhz no wait states assumed)

488 microseconds

logic:

1) find n = int(arg/ln 2).  this is added to the

mantissa at the end.

2) reduce argument to range by finding

arg = mod(arg, ln 2).

3) derive exp(arg) with cordic loop.

4) add n to exponent giving result.

L. FFPFPA -- Floating-Point to ASCII

See FORMATS section for format.

notes:

1) the binary base 10 exponent is returned in d7 to

facilitate conversions to other formats.

2) even though eight digits are returned, the precision

available is only 7.167 digits. rounding should be

performed when less than eight digits are actually

utilized in the mantissa.

3) the stack is lowered by 14 bytes by this routine.

the return address to the caller is replaced by a

portion of the results.

time: (8mhz no wait states assumed)

330 microseconds converting the sample float value

of 55.55 to ascii.

M. FFPFPBCD -- Floating-Point to BCD

See FORMATS section for format.

notes:

1) even though eight digits are returned, the precision

available is only 7.167 digits. rounding should be

performed when less than the full eight digits are

actually used in the mantissa.

2) the stack is lowered by 8 bytes by this routine.

the return address to the caller is replaced by a

portion of the results.

3) the binary base 10 exponent is returned to facilitate

conversions to other formats

time: (8mhz no wait states assumed)

approximately 330 microseconds converting the float

value 55.55 to bcd.

N. FFPFPI -- Floating-Point to Integer

Integers of over 24 bit precision will be imprecise

note:

1) maximum size integer returned if overflow

timings:  (8 mhz no wait states assumed)

composite average 15.00 microseconds

arg = 0   4.75 microseconds

arg # 0   10.50 - 18.25 microseconds

O. FFPIPF -- Integer to Floating-Point

Integers of greater than 24 bits will be rounded

and imprecise.

timings: (8mhz no wait states assumed)

composite averate 31.75 microseconds

arg = 0   4.25          microseconds

arg > 0   13.75 - 47.50 microseconds

arg < 0   15.50 - 50.25 microseconds

P. FFPLOG -- Fast Floating-Point Logorithm

notes:

1) spot checks show errors bounded by 5 x 10**-8.

2) negative arguments are illegal and cause the

"v" bit to be set and the absolute value used instead.

3) a zero argument returns the largest negative value

possible with the "v" bit set.

time: (8mhz no wait states assumed)

times are very data sensitive with samples

ranging from 170 to 556 microseconds

Q. FFPMUL/FFPMUL2 -- Fast Floating-Point Multiply

notes:

1) multipier unaltered (d6).

2) underflows return zero with no indicator set.

3) overflows will return the maximum value with

the proper sign and the 'v' bit set in the ccr.

4) FFPMUL:

this version of the multiply has a slight

error due to truncation of .00390625 in the

least significant bit.  this amounts to an

average of 1 incorrect least significant bit

result for every 512 multiplies.

FFPMUL2:

this module is the second of the multiply

routines.  it is 18% slower but provides the

highest accuracy possible.  the error is

exactly .5 least significant bit versus an

error in the high-speed default routine of

.50390625 least significant bit due to truncation.

times: (8mhz no wait states assumed)

FFPMUL

arg1 zero            5.750 microseconds

arg2 zero            3.750 microseconds

minimum time others 38.000 microseconds

maximum time others 51.750 microseconds

average others      44.125 microseconds

FFPMUL2

arg1 zero            5.750 microseconds

arg2 zero            3.750 microseconds

minimum time others 45.750 microseconds

maximum time others 61.500 microseconds

average others      52.875 microseconds

R. FFPPWR -- Fast Floating-Point Power

notes:

1) a negative base value will force the use if its

absolute value.  the "v" bit will be set upon

function return.

2) if the result overflows then the maximum size

value is returned with the "v" bit set in the

condition code.

3) spot checks show at least six digit precision for

80 percent of the cases.

time: (8mhz no wait states assumed)

the timing is very data sensitive with test

samples ranging from 720 to 1206 microseconds

S. FFPSIN/FFPCOS/FFPTAN/FFPSINCS -- Fast Floating-Point Sine/Cosine/Tangent

functions:

ffpsin   -  sine result

ffpcos   -  cosine result

ffptan   -  tangent result

ffpsincs -  both sine and cosine (d6 - sine, d7 - cosine)

notes:

1) input values are in radians.

2) function ffpsincs returns both sine and cosine twice

as fast as calculating the two functions independently for

the same value.  this is handy for graphics processing.

2) input arguments larger than two pi suffer reduced precision.

the larger the argument, the smaller the precision.

excessively large arguments which have less than 5 bits of

precision are returned unchanged with the "v" bit set.

3) for tangent angles of infinite value the largest possible

positive number is returned ($ffffff7f). this still gives

results well within single precision calculation.

4) spot checks show errors bounded by 4 x 10**-7 but for

arguments close to pi/2 intervals where 10**-5 is seen.

time: (8mhz no wait states and argument assumed within +-pi)

ffpsin       413 microseconds

ffpcos       409 microseconds

ffptan       501 microseconds

ffpsincs     420 microseconds

T. FFPCOSH/FFPSINH/FFPTANH -- Fast Floating-Point Hyperbolics

notes:

1) an overflow will produce the maximum signed value

with the "v" bit set.

2) spot checks show at least seven digit precision.

time: (8mhz no wait states assumed)

sinh  623 microseconds

cosh  601 microseconds

tanh  623 microseconds

U. FFPSQRT -- Fast Floating-Point Square Root

notes:

1) no overflows or underflows can occur.

2) a negative argument causes the absolute value

to be used and the "v" bit set to indicate that

a negative square root was attempted.

times:

argument zero         3.50 microseconds

minimum time > 0    187.50 microseconds

average time > 0    193.75 microseconds

maximum time > 0    200.00 microseconds